# EXTERNAL COLOUR & FINISHES SCHEDULE

| PROJECT DETAILS |  | DATE | 06/03/2026 |
| --- | --- | --- | --- |
| PROJECT ADDRESS | 49 Balls head road, Waverton | DEADLINE | Prior to CDC Approval Required or BASIX Assessment |
| PROJECT NAME | BALLSH49 | CLIENT NAME | Benjamin & Jordan Sutton |

| 1 | Please enter your selection in the table below |
| --- | --- |
| 2 | Please read all the information in this document, sign and return before CDC approval to admin@horizonhomes.com.au. Please note: If not returned by this time an extension of time will be claimed on the job which could delay your build time. |

| PRODUCT | RANGE / PROFILE | COLOUR | LOCATION | IMAGES | NOTES | V |
| --- | --- | --- | --- | --- | --- | --- |
| COMMON BRICKS | PGH Common Brick |  | All External |  | Acrylic render and paint finish |  |
| MORTAR COLOUR |  | STD Cement | All External |  |  |  |
| MORTAR JOINT TYPE | Flush |  | All External |  |  |  |
| RENDER | - | Dulux Grey Pebble ½ Strength S14B1H |  |  | Brick Veneer and EPS walls |  |
| WINDOW BANDS / MOULDINGS | - | N/A |  |  |  |  |
| CLADDING | Hardies Scyon Axon 133mm Smooth finish | Monument | West elevation |  |  |  |
| ROOF TILES | N/A |  |  |  |  |  |
| ROOF SHEETING | Kliplock profile | Surfmist |  |  |  |  |
| PARAPET CAPPING | Colorbond | Surfmist |  |  |  |  |
| FASCIA | Metal fascia | Monument |  |  |  |  |
| GUTTERING | Round line Flatback half round slotted gutter | Monument |  |  |  |  |
| DOWNPIPES | 90mm Diameter PVC painted | Dulux Grey Pebble ½ Strength - S14B1H |  |  |  |  |
| EAVES LINING | - | Dulux Grey Pebble ½ Strength - S14B1H |  |  |  |  |
| TIMBER POST |  | Monument | Porch x3 Balcony x3 |  | Timber posts to the Porch and balcony 3x 180x65 (Nominal) Dressed timber post |  |
| WINDOW & DOORS (Aluminium) | Powder coated aluminium windows | Monument |  |  |  |  |
| WINDOW HARDWARE (Locks, Winders etc) | Black |  |  |  |  |  |
| OPTION 1 EXTERNAL DOOR (Other – Aluminium) | Hinged door | Monument | Laundry Door |  | A doggy door can be installed in the glass of an aluminium door if this option is preferable |  |
| OPTION 2 EXTERNAL DOOR (Other – Aluminium) | Hume Entrance Door XF3 Translucent Glass | Dulux Grey Pebble Half S14B1H | Laundry Door |  | Colour has a LRV greater than 50 NOTE: CLIENTS HAVE SELECTED THIS TIMBER DOOR FOR THE LAUNDRY, TO ALLOW THEM TO INSTALL A ‘DOGGY DOOR’ AFTER HAND OVER NOTE: THIS COLOUR WILL ALLOW THE DOOR TO BLEND IN WITH THE WALLS |  |
| GARAGE DOOR | B&D Panelift Garage Doors Madrid Profile Woodgrain Finish | Monument |  |  | Madrid Profile Woodgrain Finish |  |
| ENTRANCE DOOR | Hume Savoy SX45 Entrance Door Sliced Pacific Mample (SPM) (2340x1200x40) | Clear Stain Clear glass | Front door |  | Colour has a LRV greater than 50 |  |
| SIDELIGHTS (If applicable) | N/A |  |  |  |  |  |
| BALUSTRADING (to Front Façade) | 12mm Toughened Glass with U-Channel | Powder coated Aluminium Monument | Balcony |  | Powder coated aluminium with 25yr powder from Interpon or Dulux due to salt water proximity. |  |
| EXTERNAL STAIRS (South elevation) |  | Concrete stairs Tile finish |  |  | Refer to tile selections for the tiles in this area |  |
| EXTERNAL STAIRS BALUSTRADING (South elevation) | External stairs | Powder coated Aluminium Monument | Panaroma Style |  | Powder coated aluminium with 25yr powder from Interpon or Dulux due to salt water proximity. |  |
| EXTERNAL VENETIAN BLIND |  | Monument | West elevation Outside Guest bedroom (W20) |  | PA #15 - LLV80 Motorised exterior blind to the First Floor – Guest room |  |
| PRIVACY SCREEN | Powder coated aluminium 70 x 16mm slat screen | Powder coated Aluminium Monument | Balcony |  | Powder coated aluminium with 25yr powder from Interpon or Dulux due to salt water proximity. |  |
| VERGOLA |  | Surfmist |  |  | Provisional Allowance #8 – Details TBC further in the process S&I Lifestyle Louvres 200mm motorised opening roof complete with wind sensor |  |

COMMENTS & QUESTIONS:

CLIENT ACKNOWLEDGEMENT

I/We have met with Horizon Homes representative and agree to the selection as made above. We are aware that any changes made at a later date may incur costs and an extension of time which could delay the build.

I/We acknowledge that the choice of suppliers is at the Builders discretion and that the Builder shall ensure that the colours chosen will be the same as or similar to those chosen in the selection form. In the event that a specified material in unavailable due to change of supplier the owner agrees to make any required reselection and subsequent variation to substitute the material.

I/We acknowledge that in regards to our brick selection that there is a natural variation in fired clay products affecting their colour, shade, texture & composition. We are aware that the brick chosen, may have variations in colour, shade, texture & composition which are inherent in fired clay products.

| Signature: |  | Date: |  |
| --- | --- | --- | --- |
| Name: |  |  |  |
| Signature: |  | Date: |  |
| Name: |  |  |  |